Bryan Takemoto

CPE301 – SPRING 2018

# Design Assignment 4

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 1 | COMPONENTS LIST |  |  |
| 2. | TASK 1 [DC MOTOR] SCHEMATIC AND BREADBOARD |  |  |
| 3. | TASK 1 [DC MOTOR] C CODE |  |  |
| 4. | TASK 1 [DC MOTOR] FLOW CHART |  |  |
| 5. | TASK 2 [STEPPER MOTOR] SCHEMATIC AND BREADBOARD |  |  |
| 6. | TASK 2 [STEPPER MOTOR] C CODE |  |  |
| 7. | TASK 2 [STEPPER MOTOR] FLOW CHART |  |  |
| 8. | TASK 3 [SERVO MOTOR] SCHEMATIC AND BREADBOARD |  |  |
| 9. | TASK 3 [SERVO MOTOR] C CODE |  |  |
| 10. | TASK 3 [SERVO MOTOR] FLOW CHART |  |  |
| 11. | VIDEO LINKS TO ALL TASKS |  |  |

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

Switch

Capacitors and Resistors

DC Motor

Stepper Motor

Servo Motor

Potentiometer

- TASK 1 [DC MOTOR] SCHEMATIC AND BREADBOARD

- TASK 1 [DC MOTOR] C CODE

#include <avr/io.h>

#include <stdint.h>

#include <avr/interrupt.h>

int main(void)

{

volatile uint16_t ADCvalue;

DDRD |= (1 << 6);					// PD.6 is output [PWM]

PORTD |= (1 << 2);					// Pull up the resistor for INT0

EIMSK |= (1 << INT0);				// Enable interrupt for INT0

EICRA |= 0x2;						// Trigger on falling edge

// ADC settings

ADMUX |= (1 << REFS0);	// AVcc with external capacitor at AREF pin

ADCSRA |= (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0);	// Set prescalar to 128

ADCSRA |= (1 << ADEN);	// Enable ADC

// PWM Settings

DDRD = (1 << 6);					// Set OC0A as output (PD6)

TCCR0A |= (1 << COM0A1) | (1 << COM0A0);		// Inverting mode

TCCR0A |= (1 << WGM01) | (1 << WGM00);		// Fast PWM mode

TCCR0B |= (1 << CS02) | (1 << CS00);		// Fosc/1024

sei();

while (1)

{

ADCSRA |= (1 << ADSC);			// Start conversion

while((ADCSRA & (1 << ADIF)) == 0);	// Wait for conversion

ADCvalue = ADC >> 2;

ADCvalue = 0xFF - ADCvalue;

OCR0A = ADCvalue;				// Adjust motor speed

}

return 0;

}

// Switch to turn on and off the motor

ISR(INT0_vect)

{

TCCR0A ^= (1 << COM0A1) | (1 << COM0A0);	// Turn off motor

EIFR = (1 << INTF0);				// Clear interrupt flag

}

- TASK 1 [DC MOTOR] FLOW CHART

- TASK 2 [STEPPER MOTOR] SCHEMATIC AND BREADBOARD

- TASK 2 [STEPPER MOTOR] C CODE

#include <avr/io.h>

#include <stdint.h>

#include <avr/interrupt.h>

volatile uint8_t stepperPos;

volatile uint16_t ADCvalue;

int main(void)

{

// Port initialization

DDRB |= 0x0F;

PORTB = 0x00;

// ADC settings

ADMUX |= (1 << REFS0);

ADCSRA |= (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0);

ADCSRA |= (1 << ADEN);

// Timer1 settings

TCCR1B |= (1 << WGM12);		// CTC mode

TCCR1B |= (1 << CS11);		// Prescalar of 8

TIMSK1 |= (1 << OCIE1A);		// CTC interrupt

TCNT1 = 0x00;

OCR1A = 65535;

// Initialize stepperPos

stepperPos = 0x06;

sei();

while (1)

{

ADCSRA |= (1 << ADSC);				// Start conversion

while((ADCSRA & (1 << ADIF)) == 0);		// Wait for conversion

ADCvalue = ADC << 5;

}

return 0;

}

// CTC ISR

ISR(TIMER1_COMPA_vect)

{

OCR1A = 65535 - ADCvalue;		// Speed of the stepper

if(ADCvalue > 32)

{

switch(stepperPos)		// Set next state of stepper

{

case 0x06:

stepperPos = 0x0C;

break;

case 0x0C:

stepperPos = 0x09;

break;

case 0x09:

stepperPos = 0x03;

break;

case 0x03:

stepperPos = 0x06;

break;

default:

break;

}

PORTB = stepperPos;

}

else

PORTB = 0x00;

TIFR1 |= (1 << OCF1A);	// Clear flag

}

- TASK 2 [STEPPER MOTOR] FLOW CHART

- TASK 3 [SERVO MOTOR] SCHEMATIC AND BREADBOARD

- TASK 3 [SERVO MOTOR] C CODE

#include <avr/io.h>

#include <stdint.h>

int main(void)

{

// ADC settings

ADMUX |= (1 << REFS0);		// AVcc with external capacitor at AREF pin

ADCSRA |= (1 << ADPS2) | (1 << ADPS1) | (1 << ADPS0);	// Set prescalar to 128

ADCSRA |= (1 << ADEN);					// Enable ADC

// PWM Settings

DDRB = (1 << 1);					// Set OC1A as output (PB1)

ICR1 = 5000;						// Set top

TCCR1A |= (1 << COM1A1) | (1 << COM1B1);		// Set OC1A high on compare

TCCR1A |= (1 << WGM11);

TCCR1B |= (1 << WGM12) | (1 << WGM13);		// Fast PWM Mode

TCCR1B |= (1 << CS11) | (1 << CS10);		// Clocked divided by 64

while (1)

{

ADCSRA |= (1 << ADSC);			// Start ADC conversion

while((ADCSRA & (1 << ADIF)) == 0);	// Wait for conversion

OCR1A = (ADC/3) + 200;			// Assign ADC value to PWM

}

}

- TASK 3 [SERVO MOTOR] FLOW CHART

- VIDEO LINKS TO ALL TASKS

Task 1: https://youtu.be/eb3BTntd4Lc

Task 2: https://youtu.be/4yAEPqhDWnU

Task 3: https://youtu.be/4R--y87nlpA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Bryan Takemoto